Q. No. 1: Basic Concepts

1. List down some of the basic services provided by the operating systems.

Answer: Process management, Memory management, File system management, Device management, User interface, Security, Networking

2. What is protection in the operating systems? Explain how operating system provide it.

Answer: Protection limits user access to system resources through hardware and software controls like privileged modes, access control lists, and memory protection units. These measures ensure system stability, security, and reliability by preventing unauthorized access to system resources.

3. Operating systems provide abstraction to various objects, explain how it is useful as an application developer.

Answer: Operating systems provide abstraction to various objects such as hardware, memory, and files, which makes it easier for application developers to write code without worrying about the underlying details. This makes development faster and more efficient.

4. What is an interrupt vector?

Answer: A computer system's interrupt service routines (ISRs), which are used to handle various types of interruptions, are listed in a data structure called an interrupt vector. When it occurs, the processor consults the interrupt vector to decide which ISR to run.

5. Briefly explain simple base and bound address translation scheme.

Answer: It is a memory management method in which a process is conferred access to a contiguous block of physical memory, with the operating system ensuring memory protection by maintaining the base and bound values. It is efficient but less flexible and may result in memory space wastage.

Q. No. 2 Processes

1. Briefly explain process control block and list down what information are stored in it.

Answer: PCB is a data structure. The operating system uses it to control processes and keep track of their attributes, including state, priority, process ID, CPU registers, and memory utilization.

2. What is a context switch? What operations are done in a context switch? How much time is spent in a context switch typically?

Answer: It involves saving a process's current state and then restoring another process's stored state. The CPU registers, program counter, and other process-specific data are put in the process control block during this procedure. It usually takes a few microseconds to accomplish a context switch.

3. Identify what mode will be switched upon these events:

a. Some system call issued​​[user mode to kernel mode to]

b. Interrupt is generated​​[user mode to kernel mode to]

c. Exception generated​​[user mode to kernel mode]

d. Returned from interrupt service​[kernel mode back to the user mode]

4. How many new processes are created from the following code? Support your answer with arguments.

int main(void) {

for (int i = 0; i < 3; i++)

pid_t pid = fork();

return 0;

}

Answer: 7 processes are created. In the first iteration, one child process is created by parent, resulting in a total of 2 processes. In the second, two child processes are created, resulting in a total of 4. In the third one, three child processes are created, resulting in a total of 7 processes.

5. Answer True/False

a. A process is a passive entity, while a thread is an active component. ​FALSE

b. Timer interrupts doesn’t go to priority encoder before they are received to CPU.​FALSE

c. A process creates other child processes, so a hierarchy of processes are formed in a system. TRUE

d. If fork fails, it returns a positive value. FALSE

Q. No. 3. Threads

1. Briefly explain pthread_join function. Explain why you would be using this function.

Answer: A thread's exit state is obtained by using the pthread_join function, which is used to wait for a thread to finish. This function comes in handy when we want to make sure a thread has finished before moving on to the next steps in the main program.

2. Consider the following code expressed as two threads, and list down possible values of X, Y when both thread finish the execution. Explain your answer as well.

Thread A​​​​​​Thread B

X = 8​​​​​​​X = 2

Y = X*2​​​​​​​Y = X+4

Answer: If Thread A executes first, then X = 8 and Y = 16. Then, Thread B will execute and X = 2, and Y = 6. Or Thread B executes first, then X = 2 and Y = 6. Then, Thread A will execute and X will be 8, and Y will be 16. It show the importance of synchronization mechanisms in multi-threaded programs

3. What are the information stored in a Thread Control Block (TCB)?

Answer: Thread ID, program counter, register set, scheduling information, thread state, and stack pointer are some of the pieces of information stored in a Thread Control Block (TCB).

4. Define mutual exclusion, and discuss why it’s necessary in cooperating threads.

Answer: A concurrent programming method called mutual exclusion makes sure that only one thread at a time can access a shared resource. Data races—which can result in unpredictable behavior and program crashes—must be avoided as a result of this.

5. Answer True/False

a. If a thread is waiting for an I/O to finish, the OS marks it as BLOCKED TRUE

b. Threads are light-weight as compared to processes. TRUE

c. Threads do not share anything within a process. FALSE

d. Each thread maintains its own stack. TRUE

Q. No. 4. Scheduling.

1. Briefly explain the priority scheduling algorithm, and discuss the problems it might face.

Answer: It assigns a priority value to each process and schedules processes with higher priority. Higher priorities are scheduled before those with lower priority. It may lead to starvation for lower priority processes because it may never get scheduled.

2. Consider the following jobs, and evaluate average waiting time, when the jobs appear in best order under FCFS.

P1 = 20, P2 = 5, P3 = 40, P4 = 80, P5 = 60

Answer:

P1: 0

P2: 20

P3: 25

P4: 65

P5: 145

Average waiting time = (0+20+25+65+145)/5 = 51 units

3. Imagine a process P, which requires CPU all the time, and a process Q which repeatedly requires CPU for 25ms, and then goes to disk operation for 75ms. Calculate effective disk utilization under round robin scheme with a time quantum 50ms.

Answer:

Out of each 50ms time quantum, Q will execute for a total of 50ms x (2/4) = 25ms. Since process Q requires CPU for 25ms and disk operation for 75ms, the disk utilization would be (75/(25+75)) x 100% = 75%.

4. Briefly explain SRTF algorithm, and comment on its strengths and weaknesses.

Answer: Shortest Remaining Time First is a CPU scheduling algorithm that selects the process with the smallest remaining burst time for execution.

Strengths: it provides a fast response time for short jobs and reduces average waiting time and turnaround time compared to other scheduling algorithms.

Weaknesses: it can cause starvation for longer processes, requires knowledge of the remaining time for each process, and it can result in high overhead due to frequent preemption of processes.

5. Answer True/False

a. If the time quantum of round robin is infinite, it behaves like SRTF. FALSE

b. Throughput of the system is defined as all the processes should be given equal amount of time. FALSE

c. SRTF always produces optimal scheduling solution. TRUE

d. Lottery scheduling suffers from convoy effect. FALSE